# Day 1 – 16/06/2026

### Wheel Spins

- Input: Serial

- Output: LCD Display

- Chaos: Evil Wheel

- Evil (if applicable): Must use Serial Communication

### Objective

- Have the Pot reading outputted onto the serial monitor and the LCD

### Circuit Diagram and Pin table

Potentiometer

| Part | Pin Num(s) | Type |
| --- | --- | --- |
| LCD | VSS to ground, VDD to 5V, SCK to A4 and SDA to A5. | VSS is ground, VDD is 5V (power), SCK and SDA are clock and data respectively |
| Potentiometer | Left leg to ground, Middle to A0, right to 5V (reversed direction because knob is turning away). | Left is GND, Middle is Wiper strip, right is 5V power. |

### Code notes

- Initialise variables

- Combine code from Day 003 into code.

- Utilise a password protected screen unlocked by the RFID.

- LCD alerts system when message played.

### Problems

- Vibration pin had to move from D10 to D2 due to RC522 SPI conflict

- LCD wiring swapped (VSS/VDD reversed, SDA/SCL swapped) — fixed manually

- Long compile time due to multiple heavy libraries

### Learning

- SPI devices claim specific pins — check conflicts before wiring

- I2C LCD is finnicky with pin order and VCC/GND labelling

- Test each component individually before combining

# Day 2 – 17/06/2026

### Wheel Spins

- Input: Keypad

- Output: Servo

- Chaos: Circuit must still work after being replugged in

- Evil (if applicable): Non applicable

### Objective

- Input a number on keypad (1 through 9 and 0).

- Servo moves to designated angle. (1 being 10, 9 being 90, 0 being 0).

- Pressing any other button that is not 1 through 9 or 0 resets it back to 0 anyways.

### Circuit Diagram and Pin table

Potentiometer

| Part | Pin Num(s) | Type |
| --- | --- | --- |
| Keypad | Left most pin to 2, then 3,4,5,6,7,8 and rightmost pin at \9 | All Pins within a matrix. |
| Servo | Left leg to ground, middle to 5V and right to pin 10.w | Left is GND, Middle is 5V power and right is signal for motor. |

### Code notes

- Diagnostic sketch of the Keypad

- Get the Keypd variables matrix.

- Get the inputted keypad variables and get them to move the servo.

### Problems

- Mapping out keys for the keypad.

- Nothing else really.

### Learning

- Sometimes for the keypad, not using the keypad.h is easier. The non matrix version was way easier to understand.

# Day 3 – 18/06/2026

### Wheel Spins

- Input:  Button

- Output: Buzzer

- Chaos: must use functions

- Evil (if applicable):  N/A

### Objective

- Use the button to make a buzzer system using functions.

- Short press has a single beep.

- Long press does my name in morse code (JACK)

### Circuit Diagram and Pin table

Potentiometer

| Part | Pin Num(s) | Type |
| --- | --- | --- |
| Buzzer | Top from Pin 10, bottom to ground | Top is a signal to the Buzzer, bottom is ground. (output) |
| Button | A side from pin 2, Exit to ground. | Top is a button signal (input), bottom is ground. |

### Code notes

- J is  dot dash dash dash

- A is dot dash

- C is dash dot dot dash

- K is dash dot dash

- Define dot and dash functions and then use them in beep function.

### Problems

- Finding out what my name was in morse code was the hardest part, after that, coding the subfunctions and lining them up was easy.

### Learning

- Learnt how to code a buzzer and learnt it was quite loud.

# Day 4 – 19/06/2026

### Wheel Spins

- Input:  Vibration Switch

- Output: RGB LED wheel

- Chaos: Evil

- Evil (if applicable):  Smallest Wiring space possible

### Objective

- Vibration sensor senses vibration

- RGB wheel turns on, showing colours.

### Circuit Diagram and Pin table

Potentiometer

| Part | Pin Num(s) | Type |
| --- | --- | --- |
| Vibration sensor | Left to D10, Right to GND | Left is a vibration signal, right is ground. |
| RGB LED wheel | S to D6, V to 5V, G to GND | S is a signal, V is voltage, G is Ground. |

### Code notes

- Vibration detection system starts additive loops and alerts serial.

- I % NUM_LEDS variable wraps from 0 to 8 then back to 0.

- Randomization picks 0 through 255  and uses it in HSV colour to input random colours.

- 3 times wrap around then it turns off.

### Problems

- Wiring up and finding the correct component (it looks strikingly similar to a capacitor)

- Sorting out the HSV colour randomisation part (and integrating Arrays into it).

- Understanding the 0 255 HSV colour hue was a bit confusing at first.

### Learning

- Learnt how to randomise the colours on a colour wheel.

- Learnt how the hues on the component worked.

- Learnt how to use a vibration sensor.

# Day 5 – 20/06/2026

### Wheel Spins

- Input:  3 Push Buttons

- Output: Buzzer

- Chaos: Evil

- Evil (if applicable):  no millis or delay.

### Objective

- Cooler morse code machine.

- Button one does Line beep.

- Button 2 does short beep.

- Button three is infinite.

- Buzzer is off at start.

### Circuit Diagram and Pin table

Potentiometer

| Part | Pin Num(s) | Type |
| --- | --- | --- |
| Buttons | Left to D2,3,4. Right to GND. | Left are Arduino signal pins. Right are GND. |
| Buzzer | Left to Pin 8, right to ground. | + is input form Arduino, - is ground. |

### Code notes

- Simple one today.

- Define the main variable lengths.

- Use micros (because of  the evil challenge.

### Problems

- Learning how to use micros was confusing at first, but its just really a slightly altered version of Millis, so it made sense.

### Learning

- Relearnt how to utilise 3 buttons at once.

- Leant how to use micros.

# Day 6 – 21/06/2026

### Wheel Spins

- Input:  Joystick

- Output: Servo

- Chaos: Cannot repeat jumper wire colour.

- Evil (if applicable):  N/A.

### Objective

- Use a servo motor x directions to move a servo motor clockwise and counterclockwise.

### Circuit Diagram and Pin table

Potentiometer

| Part | Pin Num(s) | Type |
| --- | --- | --- |
| Joystick | GND to GND 5V to 5V, VRx to A0. Rest unwired ( no need ) | GND is ground, 5V is power, A0 is analog horizontal input from the joystick. |
| Servo | GND is GND, 5V is 5V, D9 is signal. | Signal for output on d9, GND for Ground, 5V for power. |

### Code notes

- Very single code today.

- Utilise 0 - 1023 analog and map to the range of the servo, then write as the new angle.

### Problems

- Finding out the 0-1023 was the hardest part.

- Other than that, simple code.

### Learning

- Wiring up parts is easier now im more used to it, after doing daily activities.

- Joystick range is between 0 and 1023, and can be mapped on x or y axis, or both.

# Day 7 – 22/06/2026

### Wheel Spins

- Input:  IR sensor.

- Output: LED

- Chaos: Have to utilise Relay part.

- Evil (if applicable):  N/A.

### Objective

- Make the Light on the board flash when the IR sensor detects motion.

### Circuit Diagram and Pin table

Potentiometer

| Part | Pin Num(s) | Type |
| --- | --- | --- |
| HC-SR501 IR Sensor | GND to GND, 5V to 5V, S to D2 | GND is ground, 5V is power, D2 is digital motion signal input |
| Relay | GND to GND, VCC to 5V, IN to D3 | GND is ground, VCC is power, D3 is digital control signal |
| LED | Long leg to NC on relay, short leg to resistor to GND | resistor to GNDNC is the switched output from relay |
| Resistor 220 | Between LED short leg and GND | Current limiting for LED |
| Relay COM | COM to 5V | Common terminal, provides power to LED circuit |

### Code notes

- Used flash delay arrays to store the 5 interval values that make the sensor flash faster each time (although it was hard to see)

- Used Millis so the motion detection was more consistent (as opposed to delay).

### Problems

- Adjusting IR sensor sensitivity was finnicky.

- In built trigger delay on the IR sensor was a pain to be adjusted.

### Learning

- I understand how an IR sensor works and how to adjust its values.

- I used a relay for the first time, and I understand why you’d use it, since they act as sort of voltage switches that allow the Arduino to access more complex components that would short it otherwise.

# Day 8 -  23/06/2026

### Wheel Spins

- Input: 3 Buttons

- Output: Speaker

- Chaos: Can only upload ONCE

- Evil (if applicable): N/A

### Objective

Make a speaker that plays C,E and G tones.

### Circuit Diagram and Pin table

| Part | Pin Num | Type |
| --- | --- | --- |
| Speaker | + to D9, GND to GND | D9 is audio signal, GND is GND |
| C button | Left to D2, Right to GND | D2 is input signal, right is GND. |
| E button | Left to D3, Right to GND | D3 is input signal, right is GND. |
| G button | Left to D4, Right to GND | D4 is input signal, right is GND. |

### Code notes

- Define note values (look up values for notes online. No room for troubleshooting as I can only upload ONCE.

- Simple if else if function to make it play notes.

### Problems

- Utilised a new board connection as the ground connection was dodgy in the last days project.

- Slight difficulty finding the tone values for the system.

- Only uploading once made me recheck my code multiple times to make sure nothing was wrong before I uploaded it. Usually I upload and check for errors after.

### Learning

- Learnt how tone() works.

- How to use PWM for sound frequencies.

# Day 9 -  24/06/2026

### Wheel Spins

- Input: Keypad

- Output: Speaker

- Chaos: Left Side Board Only.

- Evil (if applicable): N/A

### Objective

Create a Keypad that controls notes, arpeggio, scales etc.

### Circuit Diagram and Pin table

| Part | Pin Num | Type |
| --- | --- | --- |
| Speaker | + to D9, GND to GND | D9 is audio signal, GND is GND |
| Keypad | Left most pin to 2, then 3,4,5,6,7,8 and rightmost pin at \9 | All Pins within a matrix. |

### Code notes

- Define note frequencies as macros for all sharps and naturals across octaves 4 and 5. Cross referenced values online.

- Manual keypad scan function instead of library — rows driven LOW one at a time, columns read with INPUT_PULLUP.

- Non-blocking melody playback using millis() so keypad stays responsive during songs.

- startMelody() and updateMelody() functions handle all melody state. make it play notes.

### Problems

- keypad library dropped keys intermittently, switched to manual scanning.

- Some melodies sounded wrong on first attempt due to incorrect note durations — had to retune timing by ear.

- Pause/resume required careful millis() bookkeeping to avoid notes getting cut short on resume

### Learning

- Learnt how to write melodies into keypad.

# Day 10 – 25/06/2026

### Wheel Spins

- Input:  Photoresistor

- Output: Buzzer

- Chaos: 1 upload attempt .

- Evil (if applicable):  N/A

### Objective

- Pitch of the buzzer heightens the brighter the light it detects is. Constant sound

### Circuit Diagram and Pin table

Potentiometer

| Part | Pin Num(s) | Type |
| --- | --- | --- |
| Buzzer | Top from Pin 10, bottom to ground | Top is a signal to the Buzzer, bottom is ground. (output) |
| Photo resistor | A side to 5V, B side to A0 and GND through 10k resistor. | A side is power, B side is GND and analog input signal. |

### Code notes

- Simple code, normal void loop function.

### Problems

- No troubleshooting meant I had to account more many variables on the first attempt, I lowered frequency from 2000 hz max to 1000 hz max to make sure the system wasn’t overwhelmingly harmful to listen to.

- After uploading, there is a background noise that is quite shrill, further uploads would let me edit it further and improve on it.

### Learning

- Learnt how to do tones on buzzer.

# Day 11 – 26/06/2026

### Wheel Spins

- Input: RFID Reader / Vibration.

- Output: LCD / RGB LED Wheel

- Chaos: Combine with previous project.

- Evil (if applicable):  N/A

### Objective

- RFID unlocks system. LCD indicates this.

- When vibration is detected, LCD indicates detection, spinning the RGB LED wheel.

### Circuit Diagram and Pin table

Potentiometer

| Part | Pin Num(s) | Type |
| --- | --- | --- |
| Vibration sensor | Left to D3, Right to GND | Left is a vibration signal, right is ground. |
| RGB LED wheel | S to D6, V to 5V, G to GND | S is a signal, V is voltage, G is Ground. |
| LCD | VSS to ground, VDD to 5V, SCK to A4 and SDA to A5. | VSS is ground, VDD is 5V (power), SCK and SDA are clock and data respectively |
| RFID Reader | SDA to D10, SCK to D13, MOSI to D11, MISO to D12, IRQ not connected, GND to GND, RST to D9 , 3.3V to 3.3V | SDA is chip select, SPI clock keeps both devices together, MOSI is data from Arduino, MISO si data to Arduino, IRQ is interrupt Pin (which im not using), GND is ground, RST is the reset pin, 3.3V is power. |

### Code notes

### Problems

- No troubleshooting meant I had to account more many variables on the first attempt, I lowered frequency from 2000 hz max to 1000 hz max to make sure the system wasn’t overwhelmingly harmful to listen to.

- After uploading, there is a background noise that is quite shrill, further uploads would let me edit it further and improve on it.

### Learning

- Learnt how to do tones on buzzer.

# Day 12 – 27/06/2026

### Wheel Spins

- Input: Serial

- Output: Buzzer (somehow?)

- Chaos: Write PsuedoCode

- Evil (if applicable): N/A

### Objective

- Turn the potentiometer and the delay between beeps decreases.

### Circuit Diagram and Pin table

Potentiometer

| Part | Pin Num(s) | Type |
| --- | --- | --- |
| Buzzer | D9 to positive, GND to GND. | D9 is power and signal, GND is GND> |
| Potentiometer | Left leg to ground, Middle to A0, right to 5V (reversed direction because knob is turning away). | Left is GND, Middle is Wiper strip, right is 5V power. |

### Code notes

- See Pseudo Code

### Problems

- Potentiometer GND and 5V legs were inverted.

- Issues with mapping potentiometer, moved to smaller physical range to get a solid set of values.

### Learning

- Learnt how to write pseudo code.

- Learnt that sometimes potentiometers have limiting physical values. That or mine is broken. Either way it works.

# Day 13 – 28/06/2026

### Wheel Spins

- Input: Accelerometer

- Output: LCD

- Chaos: Comment. Every. Line. ( :/ )

- Evil (if applicable): N/A

### Objective

- Display LCD X, Y and Z direction acceleration data on LCD.

### Circuit Diagram and Pin table

Potentiometer

| Part | Pin Num(s) | Type |
| --- | --- | --- |
| MPU 6050 | VCC to 3.3V, GND to GND, SDA to A4, SCL to A5. | VCC is power, GND is ground, SDA is data input 1, SCL is data input 2 (clock). |
| LCD | VSS to ground, VDD to 5V, SCK to A4 and SDA to A5. | VSS is ground, VDD is 5V (power), SCK and SDA are clock and data respectively |

### Code notes

- Start MPU and LCD.

- Loop X/Y/Z values, Display and repeat.

### Problems

- Potentiometer GND and 5V legs were inverted.

- Issues with mapping potentiometer, moved to smaller physical range to get a solid set of values.

### Learning

- How to wire and work the MPU.

- How the MPU interprets values? (had to change it to SI units in the code)

- How the SDA and SCL work with multiple parts.

# Day 14 – 29/06/2026

### Wheel Spins

- Input: Joystick

- Output: LED light

- Chaos: Use digital inputs only

- Evil (if applicable): N/A

### Objective

- Light up the Led each time the button digital input is detected on the Arduino (when the joystick is clicked).

### Circuit Diagram and Pin table

Potentiometer

| Part | Pin Num(s) | Type |
| --- | --- | --- |
| LED | Left to 5V, Right to D13, 220 ohm resistor and ground. | 5V is power, GND is ground, 220 is resistance for light to work, D13 is input pin. |
| Joystick | 5V to 5V, GND to GND, SW to Pin D2 | VSS is ground, 5V is power, SW is digital pin input. |

### Code notes

- Get digital read to detect  joystick press, and digital write to turn LED on and off.

- Input pullup for button reading high normally.

### Problems

- Had to check wiring due to a small issue with the Led not being pressed down properly .

### Learning

- Learnt that the joystick had a digital push button inside of it.

- Learnt that the joystick has multiple different smaller functioning parts.

# Day 15 – 30/06/2026

### Wheel Spins

- Input: Photoresistor

- Output: Speaker

- Chaos: Evil wheel.

- Evil (if applicable): No while loops.

### Objective

- Have the Pot reading outputted onto the serial monitor and the LCD

### Circuit Diagram and Pin table

Potentiometer

| Part | Pin Num(s) | Type |
| --- | --- | --- |
| LDR | Left to 5V, right to A0, 10k resistor and GND. | 5V is power, A0 is input, resistor and GND are groundd |
| Speaker | Left to D8, right to GND. | D8 is output, GND is Ground. |

### Code notes

- Simple Main function loop.

### Problems

- I improved on my previous design by recoding the old day 10 code to be more succinct, and made sure my wiring was all in place properly, which made the photoresistor more reliable.

### Learning

- Simpler photoresistor code leads to less bugs and makes the component more reliable.

# Day 16 – 1/07/2026

### Wheel Spins

- Input: Ultrasonic ranging module

- Output: LED

- Chaos: Quadruple Output.

- Evil (if applicable): N/A

### Objective

- Ultrasonic ranging module detects ranges, when it detects an object 20,15,10 and 5 m away, a new LED lights up.

### Circuit Diagram and Pin table

Potentiometer

| Part | Pin Num(s) | Type |
| --- | --- | --- |
| Ranging Module | VCC to 5V, Trig to D3, Echo to D2, GND to GND. | VCC is power, Trig sends sound, echo listens to return of sound, GND is ground. |
| LED’s | Each to 5V on left, on right D4,5,6,7 then 220 ohms resistor then ground. | 5V is power, GND is ground, 220 is resistance for light to work, D4,5,6,7 are input pin. |

### Code notes

- Tested basic ultrasonic and LED parts first.

- Piggybacked off code tests to make a system that follows the objective.

### Problems

- Mixed echo with Trig, which caused issues. Fixed by bug fixing.

- Wiring issues with LED’s but was fixed.

- Linking the 2 codes together and getting the raw data from the ultrasonic sensor was buggy but was fixed.

- Lights were flashing for some reason, added in duration function and cm to make system consistent and changed the Int variables, which seemed to stop flashing.

### Learning

- Learnt groundworks of ultrasonic component

- Learnt how to wire more than 1 LED in sequence.

- Learnt how to convert sensor to work in cm.

# Day TEMPLATE -  XX/XX/2026

### Wheel Spins

- Input:

- Output:

- Chaos:

- Evil (if applicable):

### Objective

### Circuit Diagram and Pin table

| Part | Pin Num | Type |
| --- | --- | --- |

### Code notes

### Problems

### Learning